# Review Questions

Oct 21, 2026 · 									SATVIK SINGH

## Sensors

1. What is an IMU?

A sensor that measures movement and direction. It uses an accelerometer (measures how fast it speeds up), a gyroscope (measures how fast it rotates) and sometimes a magnetometer (measures the magnetic field).

2. 6 DOF vs 9 DOF IMU?

A 6 DOF IMU has an accelerometer and a gyroscope. A 9 DOF IMU also has a magnetometer, which tells it which way is north.

3. 3 DOF of the magnetic component?

The magnetometer measures the Earth's magnetic field in three directions: X, Y and Z. Together they show where magnetic north is.

4. What is yaw, and how does a 9 DOF IMU reduce it?

Yaw is the turning left or right around the vertical axis, which is the direction the robot faces. If only a gyroscope is used, the yaw slowly drifts because small errors keep adding up. A 9 DOF IMU uses the magnetometer's north direction to fix this drift.

5. Encoder vs IMU for the arm?

Encoders are better for joint angles because they are accurate, measure the joint directly and do not drift. An IMU is only a helper, for example to check the orientation of the arm's tip or base.

## Cameras

6. Why is a stereo camera digital?

Depth is found by matching points in two images and measuring how far apart they are. This needs pixel data that a computer can process, and only a digital camera gives that.

7. Analog vs digital camera use?

Analog cameras are good for live video with very little delay, like FPV drones and old CCTV. Digital cameras are better when the video is processed, saved or analysed, like in computer vision, robotics and high resolution recording.

## Git

8. git reset vs git revert?

Reset moves the branch back to an older commit and erases the later history, so use it only on local work. Revert makes a new commit that undoes an old one and keeps the history, so it is safe on shared branches.

9. git pull vs git fetch?

Fetch downloads the changes from the remote but does not change your files. Pull does a fetch and then merges the changes into your current branch.

10. How many levels in git config?

Three: system (for all users), global (for the current user) and local (for one repository). Local beats global, and global beats system.

## Linux & Cloud

11. What do you mean by /etc/netplan?

It is the folder in Ubuntu-based Linux that stores the network settings files, written in YAML.

12. What does /etc/netplan do in a Linux distro?

Netplan reads the YAML files in that folder and sets up the network, such as the IP address (DHCP or static), DNS and Wi-Fi. It uses a backend like systemd-networkd or NetworkManager. To apply changes, run sudo netplan apply.

13. Is a cloud service a virtual machine? Are all cloud services VMs? Differentiate between them.

No, not every cloud service is a VM. A VM is just one type of cloud service (IaaS, e.g. AWS EC2). Others are not VMs for the user, such as PaaS (Heroku), SaaS (Gmail), serverless functions and cloud storage.

Difference: a VM is a technology, a full computer simulated in software with its own OS. Cloud is a way of delivering resources: you rent computing power over the internet when you need it. Every cloud VM is a cloud service, but not every cloud service is a VM.

14. Advantages of Docker?

Docker packs an app together with everything it needs, so it runs the same everywhere. Containers are lighter and faster than VMs, separate from each other, easy to move, easy to rebuild from a Dockerfile, and easy to scale.

## Sensors & LiDAR

15. How many degrees of freedom does an IMU provide?

It depends on the sensors inside. A 6 DOF IMU has a 3-axis accelerometer and a 3-axis gyroscope. A 9 DOF IMU adds a 3-axis magnetometer.

16. What is a wheel encoder, and where is it used?

A sensor on a wheel or motor shaft that counts how much it has turned. From the counts we can tell how far and how fast each wheel moved. It is used for odometry (finding a rover's position from its wheel movement) and for controlling motor speed.

17. What is a stereo camera, and how is depth perceived from it?

Two cameras placed a fixed distance apart, like two eyes. The same point looks slightly different in each image. This difference is called disparity. It is big for near objects and small for far ones, so we can calculate depth from it.

18. What is LiDAR?

Light Detection and Ranging. It sends out laser pulses and measures how long they take to come back, which gives the exact distance to nearby surfaces. The result is a point cloud, a set of points that forms a map of the surroundings.

19. Difference between 2D LiDAR and 3D LiDAR?

A 2D LiDAR scans one flat plane, so it gives one slice of the surroundings. It is cheaper and works well for flat ground and indoor mapping. A 3D LiDAR scans many planes and gives a full 3D point cloud. It works better on uneven ground but costs more and gives a lot more data.

20. Why would a rover use both LiDAR and a stereo camera?

Each one covers the other's weak points. LiDAR gives exact distances and works in the dark, but cannot see colour or texture. A stereo camera sees colour and detail, which helps to recognise objects, but its depth is less accurate and it needs good light. Together they give reliable depth and show what the objects are.

## Linux & Python

21. What are point releases (Ubuntu sub-versions)?

Updated versions of an existing Ubuntu release. They include the bug fixes and security updates made since launch, plus support for newer hardware. The main version stays the same; you just get a newer installer.

22. Difference between Ubuntu 24.04 and 24.04.5?

Both are the same release (24.04 LTS, Noble Numbat). 24.04 is the first launch and 24.04.5 is its fifth point release, which includes all the updates since then. A fully updated 24.04 system ends up almost the same as 24.04.5.

23. Write Python code that reverses a word using string slicing.

word = "rover"

print(word[::-1])    # revor

The slice [::-1] uses a step of -1, so it reads the string from the end to the start.

24. What does .strip() do?

It gives a copy of a string with spaces, tabs and newlines removed from both ends. You can also give the characters to remove, e.g. text.strip("-"). The original string does not change, because strings are immutable.

25. Difference between an array and a list?

A Python list can hold items of any type and can grow or shrink easily. An array (from the array module or NumPy) holds items of one type only, stored compactly, so it uses less memory and is much faster for maths. NumPy arrays also allow element-wise operations, which lists do not.

## ArUco & Camera Calibration

26. What is the purpose of camera_matrix in the code? Does the camera matrix remain the same for all devices?

It describes the inside geometry of the camera: its focal length and the centre of the image. OpenCV uses it to connect 3D points in the world to pixel positions, which is needed to find a marker's pose and distance. It is not the same for all devices. Every camera, and every resolution, has its own matrix, so each camera must be calibrated.

27. Why is the orientation coefficient initialised to 0 in the code?

These are the distortion coefficients. They are set to 0 when there is no calibration file. Zero means "assume the lens has no distortion", so pose estimation can still run, but it is a little less accurate, especially near the edges of the image. After calibration, the real values replace the zeros.

28. What is the purpose of if __name__ == "__main__": in Python? Will the code inside this block execute if __main__ is changed to another name?

Python sets __name__ to "__main__" when you run the file directly, and to the module's name when the file is imported. So the block runs only when the file is run directly, and the functions can be imported elsewhere without starting the program. If "__main__" is changed to another name, the condition is never true and the block does not run.

29. How many distortion coefficients are there and what are they?

The standard model uses five: k1, k2 and k3 for radial distortion, and p1 and p2 for tangential distortion. Radial distortion bends straight lines, more near the edges of the image (barrel or pincushion). Tangential distortion happens when the lens is not perfectly parallel to the sensor.

30. Difference between .npz and .yaml files?

An .npz file is NumPy's compressed binary format for saving several arrays together. It is fast and easy to use in Python, but people cannot read it. A .yaml file is plain text, so people can read and edit it and other languages can load it, but it is bigger and slower to read.

31. What is the difference between original ArUco and 5x5 ArUco, and why do we need original ArUco?

Original ArUco is the first marker set from the ArUco library: 5x5 markers with 1024 IDs. The newer 5x5 dictionaries were made later so the markers look as different from each other as possible, which means fewer mix-ups, and they come in sizes (50, 100, 250, 1000) to match how many IDs you need. Original ArUco is kept so older markers and systems still work.

32. What functions are there in cv2.aruco?

They fall into a few groups:

Dictionaries: getPredefinedDictionary() loads a marker set.

Marker generation: generateImageMarker() makes a marker you can print.

Detection: ArucoDetector with DetectorParameters finds markers and their corners.

Drawing: drawDetectedMarkers() draws outlines around the found markers.

Boards for calibration: GridBoard, CharucoBoard and CharucoDetector.

Pose is then found with cv2.solvePnP(), using the detected corners.

33. What is cv2.Rodrigues()?

It converts between two ways of describing a rotation: a rotation vector (an axis, with its length equal to the angle turned) and a 3x3 rotation matrix. It works both ways. solvePnP() gives a rotation vector, and Rodrigues turns it into a matrix, which can then give angles like roll, pitch and yaw.

## Image Processing

34. Why use HSV and not BGR?

In BGR, colour and brightness are mixed in all three channels, so a shadow changes all three values. HSV keeps them separate: hue (the colour itself), saturation (how strong it is) and value (brightness). We can pick a colour by its hue range, which stays fairly steady when the light changes, so thresholding is easier and more reliable.

35. What does cv2.moments() do? Explain all its attributes.

It calculates sums over the pixel positions of a contour or binary image, which describe its shape. The attributes come in three groups:

Spatial moments (m00, m10, m01, m20, ...): m00 is the area, and m10/m00 and m01/m00 give the centre (centroid).

Central moments (mu20, mu11, mu02, ...): the same idea measured around the centroid, so they do not depend on position. They describe spread and orientation.

Normalised central moments (nu20, nu11, ...): these also do not depend on size. They are used for shape matching, for example in Hu moments.

36. Difference between Gaussian blur and morphologyEx()?

Gaussian blur smooths an image by replacing each pixel with an average of the pixels around it, where closer pixels count more. It reduces noise and small details, works on any image and softens edges. It is usually done before thresholding.

morphologyEx() changes the shape of regions, usually in a binary mask, using a small kernel. It is built from erosion (shrinks white areas) and dilation (grows them). Opening removes small specks of noise and closing fills small holes. It is usually done after thresholding.

In short, blur smooths the pixel values, while morphology cleans up the shapes in a mask.

37. What is the fundamental difference between Hough circles and circularity? Which is better and why?

Hough circles looks for circles in the edges of an image: each edge point votes for possible circle centres, and the strongest votes become circles. It can find partly hidden circles without needing colour, but it is slower and sensitive to its settings. Circularity checks how round an already-found blob is (1 means a perfect circle). It is fast and simple, but it needs a good colour mask first and struggles when the ball is partly hidden.

For a bright tennis ball on a live camera, circularity is usually better because it is fast enough for real time. Hough is better when colour is not reliable. Using both, with Hough confirming what the colour mask found, is also common.

38. What is the difference between methods and functions?

A function is a separate block of code that you call by name, e.g. len(x). A method is a function that belongs to an object or class and is called on that object, e.g. text.strip(). A method automatically gets the object it is called on (self) as its first argument.

39. In addition to the current code, create a separate black-and-white window showing the centre of each blue object (x, y), its position relative to the yellow tennis ball and its distance to other objects.

Done in tracker.py. A second window, "Info", is shown on a pure black background next to the camera feed. It shows each blue object's centre (x, y), its position relative to the tennis ball (dx, dy and straight-line distance), the distance between blue objects, and how far each object has moved. Distances are in cm when an ArUco marker is visible, and in pixels otherwise.